Recommended phases

Phase 1 — GPIO read + basic STA connection

Goal: ESP32 reads GPIO19 at boot, connects to a phone hotspot with hardcoded test credentials, web server works on either interface.

Files:

- Morse_Trainer.ino — add WIFI_MODE_PIN on GPIO19, read in setup(), branch

- WiFiSTA.h / WiFiSTA.cpp — new module: initWiFiSTA() with hardcoded SSID/password, connect with timeout, print IP to serial

- WiFiAP.h — add enum class WiFiOpMode { AP, STA } and getWiFiOpMode() accessor

Test: Flip switch, reboot, confirm serial shows "STA mode → connected to <SSID> at <IP>". Open browser to that IP — web UI loads. Flip switch back — AP mode works as before.

---

Phase 2 — NVS credential storage + serial commands

Goal: SSID and password stored persistently; configurable without reflashing.

Files:

- WiFiSTA.cpp — load SSID/password from NVS instead of hardcoded

- MorseCommands.cpp — add wifi ssid <name>, wifi pass <pw>, wifi show commands

- Storage.cpp — no change (existing NVS namespace handles new keys)

Test: wifi ssid MyPhone + wifi pass secret123 via serial, reboot in STA mode, confirms it connects using saved credentials.

---

Phase 3 — mDNS discovery

Goal: http://cwtrainer.local works in both AP and STA modes so the user never needs to know the IP.

Files:

- Morse_Trainer.ino — add #include <ESPmDNS.h>, call MDNS.begin("cwtrainer") after either WiFi init

Test: In both modes, browse to http://cwtrainer.local — web UI loads.

---

Phase 4 — STA robustness (reconnect + status LED)

Goal: If the phone hotspot drops or the ESP32 loses connection, it auto-reconnects. LED indicates connection state.

Files:

- WiFiSTA.h / WiFiSTA.cpp — add checkWiFiSTA() for loop(), reconnect with backoff, connection state tracking

- Morse_Trainer.ino — call checkWiFiSTA() in loop() (parallel to existing checkWiFiClients())

- LED feedback: steady = connected, slow blink = connecting/reconnecting

Test: Connect in STA mode, disable phone hotspot, watch serial — ESP32 retries. Re-enable hotspot — reconnects automatically.

---

Phase 5 — Web UI + WebSocket status polish

Goal: Phone UI shows which WiFi mode is active and the current IP address.

Files:

- MorseWebSocket.cpp — include wifi_mode, wifi_ip in status broadcasts

- Data/app.js — display AP/STA indicator and IP in header

- index.html — optional: WiFi settings panel for STA credentials

Test: UI shows "AP · 192.168.4.1" or "STA · 192.168.x.x" in the header.

---

Summary

| Phase | What | Risk | Independently useful? |
| --- | --- | --- | --- |
| 1 | GPIO + STA connect + branch | Medium (new WiFi code) | ✅ Fully functional |
| 2 | NVS creds + serial commands | Low | ✅ No more reflashing for SSID changes |
| 3 | mDNS | Very low | ✅ Quality-of-life improvement |
| 4 | Reconnect + LED | Low-medium | ✅ Essential for real-world use |
| 5 | UI polish | Low | ✅ Nice to have |

Shall I start Phase 1?

Summary of changes

| File | What changed |
| --- | --- |
| WiFiSTA.h / WiFiSTA.cpp | New. STA init using WiFi.begin(), connect with 15s timeout, IP reporting. Hardcoded credentials for testing. |
| WiFiAP.h | Added WiFiOpMode enum, setWiFiOpMode(), getWiFiOpMode(), getWiFiOpModeName(). Existing API unchanged. |
| WiFiAP.cpp | Added WiFiOpMode state + accessors. All existing AP code untouched. |
| MorseWebServer.cpp | initWebServer() now checks either AP or STA readiness. Serial output shows correct IP for whichever mode. Added wifi_mode to /api/status. |
| Morse_Trainer.ino | Added WIFI_MODE_PIN (GPIO19), pinMode with INPUT_PULLUP, reads switch before WiFi init, branches to initWiFiAP() or initWiFiSTA(). loop() guards AP-only calls behind mode check. |

Testing

- Edit WiFiSTA.cpp — change STA_SSID and STA_PASSWORD to match your phone's hotspot name and password

- AP mode — leave GPIO19 floating (or wire to 3.3V). Boot. Everything works as before

- STA mode — wire GPIO19 to GND through a switch. Enable phone hotspot. Boot. Serial shows the assigned IP. Open that IP in a browser — full web UI loads

Change summary

| File | What changed |
| --- | --- |
| WiFiSTA.h | New. Declares initWiFiSTA(), isSTAReady(), getSTAIPAddress(). |
| WiFiSTA.cpp | New. STA connect using WiFi.begin(), 15s timeout, IP/RSSI/MAC reporting. Hardcoded test credentials. |
| WiFiAP.h | Added WiFiOpMode enum + setWiFiOpMode() / getWiFiOpMode() / getWiFiOpModeName(). All existing declarations unchanged. |
| WiFiAP.cpp | Added wifiOpMode static + 3 accessor implementations (lines 19–32). All existing AP code untouched. |
| MorseWebServer.cpp | Added #include "WiFiSTA.h". initWebServer() now checks AP or STA readiness. handleApiStatus includes wifi_mode. Serial URLs use correct IP for active mode. |
| Morse_Trainer.ino | Added #include "WiFiSTA.h", WIFI_MODE_PIN (GPIO19) with INPUT_PULLUP. setup() reads switch and branches to initWiFiAP() or initWiFiSTA(). loop() guards AP-only calls behind getWiFiOpMode() check. |

Before testing

Edit WiFiSTA.cpp lines 10–11 to match your phone's hotspot:

const char* STA_SSID     = "YourPhoneName";

const char* STA_PASSWORD = "YourPassword";

Test matrix

| Switch position | Expected serial output | Web UI |
| --- | --- | --- |
| GPIO19 floating/HIGH | WiFi switch: AP → normal AP boot | http://192.168.4.1 |
| GPIO19 tied to GND | WiFi switch: STA → connects to hotspot → prints IP | http://<printed IP> |

The only change across all four files is "WiFiSTA.h" → "WiFiStation.h". The existing WiFiAP.h / WiFiAP.cpp happen to work because they use the ESP-IDF C API directly and never reference WiFiAPClass — but our STA module uses the Arduino WiFi wrapper, which triggers the name collision with the core library's WiFiSTA.cpp.